Roll-A-Shade® ZipShade® 100 surface mounted in front

Roll-A-Shade, Inc. 112 College St., Suite 411 Toronto, ON M5G 1L6

bids@rollashade.com www.rollashade.ca

2016-09-01

Product features

(text marked in red can be deleted depending on your choice)

Installation

The box is installed in front of the window frame -> Surface mounted box.

-> The finishing of the screen at the frontal side needs to be a removable profile

The electrical connection is made when installing the fabric roller (fabric roller + fabric).

Box

Dimensions: 100 mm high and 100 mm deep

The box consists of a fixed profile and a removable profile.

Profiles are made of extruded aluminium.

The side supporting end pieces of the box, which support the roller mechanism and are equipped with pins, connect the box to the side channels.

Fabric roller

Made of galvanised steel.

Recessed fabric slot limits compression of the fabric strap.

A patented conical endcap

The patented conical endcaps compensate for the larger ends of the zippers.

The fabric roller can be removed from the side with the removable profile, which will define the position of the motor on the left or right hand side.

Fabric

All screens are a single piece of fabric, except when the height is greater than the width of the fabric roller.

The fabric is manufactured horizontally.

The vertical borders are equipped with a zipper, making the fabric is windproof in the side channel.

The zipper is high-frequency welded, always on the least visible side.

Dimensions

Semi-transparent fabric:

- Max. width 4,000 mm and max. height 2,700 mm or max. width 3,000 mm and max. height 3,500 mm in 1 part (max. 10.8 m²)

Blackout fabric:

- Min. width 1,000 mm; max. width 2,000 mm and max. height 2,700 mm in 1 part (max. 5.4 m²)

Side channels

Are made of 2 extruded aluminum profiles.

Dimensions:

- Closed side channel in two parts: 35 mm W x 48 mm deep

The side channels (open & closed) are pre-drilled (first chamber).

Equipped with a nose section that allows the basic width of the profile to be kept to a minimum.

They are screw fixed directly onto the window frame / structure. No screws are visible at the side of the façade.

Along with the weighted bottom bar, provides the ideal guide when the fabric moves up and down.

The box is fixed on the side channels by means of pins in the side supporting endpieces that slide into the hollow chambers.

Each side channel has an integrated HPVC inner rail with a co-extruded, wear-resistant top coating (Smooth technology).

The HPVC inner rail is equipped with Neoprene buffer zones (60 mm long) to compensate for heavy wind loads.

The zip, which is welded to the fabric, is threaded through this HPVC inner rail, which holds the fabric in place.

When installed correctly, there is sufficient tolerance between the fabric, aluminium side channels and the HPVC inner rail to guarantee ease of use.

Weighted bottom bar

Is made of 1 extruded aluminum profile and is weighted with galvanized steel bars.

Is equipped with plastic endpieces.

Is equipped with a plastic sealing strip to seal off the sill. Available in 2 colours: black and grey

Guide system

Smooth technology

Guides the bottom bar and the fabric.

Thanks to the patented Smooth technology, the movement of the zipper in the HPVC inner rail is smooth and silent.

This intelligent HPVC inner rail is equipped with a patented, wear-resistant layer.

- guarantees a taut fabric with fewer wrinkles

- does not need yearly maintenance with a lubricant

Colour

All visible aluminium profiles (box, side channels and bottom bar) are powder-coated in the same RAL colour (60-80 µm)

The side supporting end pieces are cast aluminum and are painted in the same colour as the profiles.

Control

Electrical: using a 120 VAC tubular motor

The connection is included in the sun protection set.

Includes a cable equipped with a UV-resistant jacket.

The power supply and all wiring are included in the electrical set.

0

Warranty

10-year warranty on all coatings on the aluminium elements.

7-year warranty on the Fixscreen-technology.

- zip remains in side channel

- optimal adhesion of zip to fabric

5-year warranty on all defects arising from normal home use and regular maintenance.

5-year warranty on gloss (coatings).

5-year warranty on the electronic operating system (Somfy® motorisation & automation).

5-year warranty on the fabric collection.

Wind class

This screen meets European standard EN13561.

Guaranteed up to 130 km/h when closed.

- EN 13561:2004+A1:2008 in accordance with wind resistance class 3

- NBN EN 13561:2015, in accordance with wind resistance class 6. The norm hasn’t been published yet on a European level. The results are subject to changes.

- Wind tunnel test report 'Force Technology' institute (N° 113-25809): wind resistance guaranteed up to 127 km/h when closed (tested for a screen of 3,000 mm x 3,000 mm)

- The wind resistance depends on the screen dimensions (W x H) and is available on request