Roll-A-Shade® ZipShade® 100 SLIM Recessed Installation

Roll-A-Shade, Inc. 112 College St., Suite 411 Toronto, ON. M5G 1L6

bids@rollashade.com www.rollashade.ca

2016-09-01 Ssulm/Osmed

Product features

(text marked in red can be deleted depending on your choice)

Installation

The system is installed in front of a construction and/or integrated into a construction and can be:

-> Surface-mounted directly on a construction

-> Concealed and/or integrated into a construction

-> Standard or 'Freestanding' F without underlaying windowframe

Installation method 7A (Standard) and 7B (with reversed box and side channels).

Box

Dimensions: 150 mm high x 110 mm deep

Box design: Square (rectangular)

The box consists of 2 fixed profiles and a removable bottom profile.

Profiles are made of extruded aluminium.

The side supporting end pieces of the box, which support the roller mechanism and are equipped with pins, connect the box to the side channels.

Fabric roller

Made of galvanised steel.

Recessed fabric slot limits compression of the fabric strap.

A patented conical endcap, the motor slide and the electrical motor connector are installed on the motor side.

A patented conical endcap and a bearing slide are installed on the bearing side.

The patented conical endcaps compensate for the larger ends of the zippers.

The fabric roller can be removed downwards from the side with the removable profile, which will define the position of the motor on the left or right hand side.

Fabric

All screens are a single piece of fabric, except when the height is greater than the width of the fabric roller.

The fabric is manufactured horizontally.

The vertical borders are equipped with a zipper, making the fabric is windproof in the side channel.

The zipper is high-frequency welded, always on the least visible side.

Dimensions

Semi-transparent fabric:

- Max. width 4,500 mm and max. height 2,800 mm or max. width 3,000 mm and max. height 3,500 mm in 1 part (max. 12.6 m²)

Blackout fabric:

- Min. width 1,000 mm; max. width 2,000 mm and max. height 2,800 mm in 1 part (max. 5.6 m²)

Dimensions if 'Freestanding' F

Semi-transparent fabric (glass fibre fabric):

- Max. width 4,500 mm and max. height 2,800 mm in 1 part (max. 12.6 m²)

Blackout fabric: NOT POSIBBLE

Side channels

Are made of 2 or 3 extruded aluminum profiles.

Dimensions:

- Depth side channel in three parts (standard): 35 mm W x 110 mm deep

- Closed side channel in three parts: 35 mm W x 48 mm deep

- Coupling side channel in three parts: 58 mm W x 48 mm deep

Equipped with a nose section that allows the basic width of the profile to be kept to a minimum.

They are screw fixed directly onto the window frame / structure. No screws are visible at the side of the façade.

Along with the weighted bottom bar, provides the ideal guide when the fabric moves up and down.

The box is fixed on the side channels by means of pins in the side supporting endpieces that slide into the hollow chambers.

Each side channel has an integrated HPVC inner rail with a co-extruded, wear-resistant top coating (Smooth technology).

The HPVC inner rail is equipped with Neoprene buffer zones (60 mm long) to compensate for heavy wind loads.

The zip, which is welded to the fabric, is threaded through this HPVC inner rail, which holds the fabric in place.

When installed correctly, there is sufficient tolerance between the fabric, aluminium side channels and the HPVC inner rail to guarantee ease of use.

Weighted bottom bar

Is made of 1 extruded aluminum profile and is weighted with galvanized steel bars.

Disappears into the box.

- Dimensions and weight of the bottom bar: 46 mm H x 30 mm thick (excl. sealing strip) = 0.85 kg/lm

- Dimensions and weight of the steel bar: Ø 22 mm = 3 kg/rm if width ≤ 2,000 mm

- Dimensions and weight of the steel bar: Ø 18 mm = 2 kg/rm if width > 2,000 mm

- Dimensions and weight of the steel bar: 30 mm H x 20 mm thick = 4.7 kg/rm if 'Freestanding' F

The bar is covered with PE foam to prevent contact between the aluminium and steel.

Is equipped with plastic endpieces.

Is equipped with a plastic sealing strip to seal off the sill. Available in 2 colours: black and grey

Guide system

Smooth technology

Guides the bottom bar and the fabric.

Thanks to the patented Smooth technology, the movement of the zipper in the HPVC inner rail is smooth and silent.

This intelligent HPVC inner rail is equipped with a patented, wear-resistant layer.

- guarantees a taut fabric with fewer wrinkles

- does not need yearly maintenance with a lubricant

Colour

All visible aluminium profiles (box, side channels and bottom bar) are powder-coated in the same RAL colour (60-80 µm)

The side supporting end pieces are cast aluminum and are painted in the same colour as the profiles.

Control

Electrical: using a 120 VAC tubular motor, without manual emergency override

The connection is included in the sun protection set.

Includes a cable equipped with a UV-resistant jacket.

The power supply and all wiring are included in the electrical set.

Warranty

10-year warranty on all coatings on the aluminium elements.

7-year warranty on the Fixscreen-technology if standard;

5-year warranty on the Fixscreen-technology if 'Freestanding' F:

- zip remains in side channel

- optimal adhesion of zip to fabric

5-year warranty on all defects arising from normal home use and regular maintenance.

5-year warranty on gloss (coatings).

5-year warranty on the electronic operating system (Somfy® motorisation & automation).

5-year warranty on the fabric collection (2-year warranty for Crystal window).

Wind class

This screen meets European standard EN13561.

Guaranteed up to 130 km/h when closed.

- EN 13561:2004+A1:2008 in accordance with wind resistance class 3

- NBN EN 13561:2015, in accordance with wind resistance class 6. The norm hasn’t been published yet on a European level. The results are subject to changes.

- Wind tunnel test report 'Force Technology' institute (N° 113-25809): wind resistance guaranteed up to 127 km/h when closed (tested for a screen of 3,000 mm x 3,000 mm)

- The wind resistance depends on the screen dimensions (W x H) and is available on request

Guaranteed up to 60 km/h when closed if 'Freestanding' F (EN 13561:2004+A1:2008 in accordance with wind resistance class 3).